# Adaptive Navigation of Multirobot Systems in 2D Vector Fields

Christopher Waight

Mech Eng Department

Santa Clara University

Christopher Kitts

Mech Eng Department

Santa Clara University

Adaptive Navigation of a multi-robot system maneuvers the system of robots based solely on distributed sensor readings of the environment. This approach allows for the exploration of unknown environments and stochastic environments with moving features of interest. Research in this area primarily involves work done with scalar fields, with initial work finding extrema in vector fields. This document describes how vector fields can be constructed from simpler basis vector fields using superposition and identifies regions of interest within these environments including local extrema points, stagnation points, equi-magnitude contours, and separatrix trajectories. It recaps cluster control formulation, a centralized method of formation control for multirobot systems which is used in this research. A few control primitives which rely on purely reactive control are developed to maneuver the multi-robot system to features of interest in vector fields. These control primitives are then demonstrated in several simulated environments. The performance of each control primitive is then discussed, followed by a brief explanation of upcoming plans to demonstrate these primitives on a physical test bed.

Keywords – Adaptive Navigation, Vector Fields, Multirobot Systems, Cluster Space.

# #1. Introduction

Multirobot systems are coordinated groups of robots that often have capabilities beyond that of individual robots [1][2]. In a multirobot system, each robot offers speed, precision, repeatability, safety, and resilience to extreme environments [3]. When grouped into a multirobot system, the system has additional advantages such as redundancy, expanded coverage, and collaborative task execution, which is vital for missions ranging from search and rescue to environmental monitoring [4-5].

When the environment is known or mapped, path planning can be effectively executed using predefined navigation methods. In instances where environmental knowledge is limited, Adaptive Navigation methods update a vehicle’s desired trajectory in real time using environmental readings as inputs [28]. These methods retain the benefits of being able to withstand harsh conditions, but further enabling robots to navigate in environments dynamic or communication-restricted environment [6]. Multirobot Adaptive navigation in scalar field environments is well studied in both simulation and practice. This includes formation control using centralized and decentralized architectures, and developing control strategies for extremum seeking, contour following, and other regions of interest [7-11]. Use cases for this include tracking contamination areas, oil spills and source points [12-15], and plume front tracking [16].

Extending these methods to vector field environments is in its infancy, with most of the work done is using artificial vector fields for formation control [17-19].  There are only a few using distributed vector readings from a multirobot system in a vector field [20-23]. It has been shown in simulation that control primitives for multirobot systems exist, including methods for navigating to the fastest flowing point in a stream, navigating to the center of a vortex, sink or source, as well to the center of a sinking vortex [13]. Indoor test beds have also been developed to demonstrate a few of these primitives [22]. This extension to vector fields provides multirobot systems with new methods to navigate extreme environments in fluid flow fields such as rivers, oceans, and aerial spaces [21-24]. Moreover, this extension allows for adaptive navigation methods to be used in extraterrestrial settings autonomous navigation through non-uniform gravitational fields, magnetic fields, and thermal gradients, and other vector fields, such as during asteroid mining [25] [26].

In this paper, we extend research in vector field adaptive navigation using multirobot systems with centralized control to locating additional features of interest using newly developed control primitives.  Section 2 covers the high-level control architecture of the multi-robot system. In Section 3, the paper discusses different basis for creating vector fields using superposition and introduces a list of features of interest. Section 4 defines control primitives used to navigate to each feature of interest in a purely reactive manner. Section 5 discusses the performance and limitations of these control primitives in simulated environments. Section 6 closes this work with a few ideas for future research. Section 7 acknowledges all who made this research possible.

# #2. Adaptive Navigation Control Architecture

The controller architecture used for navigation of the multirobot system can be divided into 3 main parts: a layer with individual robot controllers for control over each robot, a layer with a cluster space controller for formation control, and an adaptive navigation layer for executing the control primitives. An overview of this architecture can be seen in figure 1.

Figure 1. Multilayered Control Architecture Block Diagram

The robots used in simulation are abstracted to a single massless point in space, and the robots are assumed to be omnidirectional. Each robot has sensors that return a single instantaneous vector reading of the environment, as well as its relative location to all the other robots.

To maintain formation, a cluster space controller is used. Cluster Space control is a technique developed by Dr. Chris Kitts of Santa Clara University. In this technique, all robot positions are described geometrically relative to a cluster frame. A generalized cluster in a 2-dimensional plane can be seen in figure 2.

Figure 2. A 3-robot cluster with cluster formation variables

For this research, a 3 robot cluster with the formulation used in figure 2 was used. The cluster values, p=q= 0.854, and beta = 4pi/35 were chosen as our cluster variables. The robot orientation pose variables are not used in this study, as the robots are assumed to be omnidirectional.

The adaptive navigation layer hosts the control primitives and the cluster shape policy. It tells the cluster space layer which shape policy to use and commands the entire cluster in a direction according to control primitive.

This architecture has been successfully demonstrated in a variety of studies, including finding local extrema and contour following in scalar fields, and in an initial study that finds extrema points in vector fields. It has also been used to maneuver two balloons in a 3D environment.

# #3. Vector Fields

## #3.1 Construction of Complex Vector Fields

The Laplace Equation, given by:

∇2 ϕ=0

Where ∇2 is the Laplace operator and ϕ  is a scalar potential function.  The gradienst of the potential functions can be described as vectors. Furthermore, the inherent linearity property of the solutions allows for superposition of solutions. This motivates the idea that solutions to the Laplace equation can be effectively superimposed to model complex vector fields. This methodology was pivotal in creating model environments for our simulations.

Figure 3 shows an example of a uniform vector field. This field could be superimposed with other uniform flows over a smaller region to create non-uniform flows, such as an increased current in the middle of a river flowing faster (see figure 4).

Figure 3. Uniform Flow Fields

Figure 4. Modelling a river with a strong centre flow

Other solutions to the Laplace equation include source features and sink features, modelled with a magnitude of r-2 and pointing in a direction away from or towards the extrema point for sources and sinks respectively (see Figure 5). Examples of these in nature include hot springs and gravitational fields. shows a source feature, in which vectors point away from the source, and the magnitude falls of like 1/r^2. If the arrows were pointing inward, it would be a sink. Examples of these in nature include hot springs and wormholes.

Figure 5. A source in a vector field

Vortices are also valid solutions to the Laplace equation. This includes both free vortices and fixed vortices. (see figure 6)

Figure 6. Vector Fields for Free Vortex

Magnetic dipoles can also be modeled as a fundamental solution the laplace equation. Alternatively, dipoles can be modlled as a source point that is located infinitesimally close to a sink point (see figure 7).

Figure 7. Dipole – Magnitudes normalized for visualization

When considering boundary conditions with the laplace equation, it is possible to model flows around a corner, such as a flow along a winding river bed, or flow around an obstacle (see figure 8)

Figure 8. Flow around a 90-degree corner

By combining vortices, sinks, sources, uniform flows, and dipoles, more complex vector fields can be created, such as a sinking vortex, a spring in a stream, or co-rotating vortices (see figures 9,10 and 11). Free and Fixed vortices can be superimposed to model a rankine vortex, which parallels the vector flows in a tornado or in a hurricane.

Figure 9: Vortex superimposed with a uniform fluid flow

Figure 10: Corotating Vortices

Figure 11: A sink superimposed with a vortex

## #3.2 Vector Fields - Regions of Interest

By observing several complex vector fields from combinations of basis vector fields, a list of regions of interest has been compiled. Table 1 describes all interesting features that have been identified, and states which combinations can be used to create them.

Local extrema points such as a sink, source, or sinking vortex have been studied before, and navigating towards them has been demonstrated in both scalar and vector fields. The extrema point can also be the location of the source of a dipole, as the source has the strongest vector magnitude.

Equi-magnitude lines can be thought of as contour lines if a contour plot is made using the magnitude information of a vector field only (see figure 12). Finding an Equi magnitude line could be useful if the individual robots have limitations such as being unable to handle stronger environmental conditions in the vector field beyond some threshold.

Another parallel to scalar field features is the fastest flow stream. In a magnitude only plot, this would appear as a crest. Finding the fastest flow is important, as it correlates to the highest flow rate in a stream for collecting debris or fish.

Stagnations points are areas where the velocity of the vector field is 0. A stagnation point in a vector field may exhibit saddle point behavior, in that it is stable in some direction, and unstable in other directions.

Figure 12: Spring in Stream - Source, Stagnation Point, Fastest Flow, Constant Flow

Another common feature in vector fields is a separatrix, a curve or trajectory that separates the phase into different dynamics. This can be seen in Figure 13. Following the Vectors below the separatrix point to a clockwise path, while the vectors above the separatrix point in a counterclockwise direction. Moving along a separatrix is analogous to traversing a ridge, as falling off the  leads to differing paths of descent..

Figure 13. Dipole Features including Equi magnitude Lines, extrema point, and a Separatrix

Moreover, combining many flows and incorporating additional flow dynamics, features such as periodic orbits, stagnant trajectories, homoclinic trajectories, and chaotic attractors can be created stable stagnation zones can be created. These features remain beyond the scope of this research. Furthermore, there may be scalar field regions of interest such as tunnels and overhangs that do not have vector field equivalents.

Table 1:  Regions of Interest in Vector Fields

| Region of Interest | Fields Where Present |
| --- | --- |
| Extrema points | Sinks Sources Dipoles Rankine Vortex Fixed Vortex Free Vortex Sink + Vortex Source + Vortex Spring in Stream Opposing Vortices Corotating Vortices |
| Equi magnitude Contours | Sinks Sources Dipoles Rankine Vortex Fixed Vortex Free Vortex Sink + Vortex Source + Vortex Spring in Stream Opposing Vortices Corotating Vortices |
| Fastest Flow Stream | Corotating Vortices Superimposed Linear Streams Spring in Stream Dipole Vortex in Stream Opposing Vortices Rankine Vortex |
| Separatrix | Dipole Vortex in stream Opposing Vortices Opposing Streams Spring in Stream Corotating Vortices |
| Stagnation Point | Spring in Stream Opposing Vortices Vortex in Stream Opposing Streams Superimposed Linear Streams |

# #4. Control Primitives

Previous researchers established two classes of primitives for autonomous navigation of multirobot systems in vector fields. [14] These have been named as the ‘vector to scalar’ technique, and the ‘direction follower technique’. These will be described briefly, and 3 new techniques will be introduced in this section.

## 4.1 Vector to Scalar Technique

In the technique, only the magnitude information from the vector readings is used. Differences in sensor readings from the individual robots are used to calculate the normal angle to the plane, and then projected to provide the direction of steepest change. Equations 2-6 show how the xi, yi,coordinates and the magnitude of the vector readings zi are used to determine the direction of travel. NB: The gradient becomes ill defined if the 3 robots formed a straight line.

[Equations 2 – 6 need to be typed]

## 4.2 Direction Follower Technique

This technique uses only the direction information from the vector readings. The direction of each vector reading is averaged by normalizing the magnitude of each vector then computing the vector sum. The direction of the resultant vector is then used to direct the multirobot system.

## 4.3 Vector Sum Technique

This proposed technique is a modification to the direction follower technique, in that the vectors are not normalized before summing. This may lead to improvements when larger vectors are more important.

## 4.4 Intersecting Vectors Technique

For each pair of robots, the intersection point of their vectors is calculated and then averaged. This guides the cluster towards, away, or around that feature based on distributed knowledge. This calculated point also provides an estimated position for the features of interest, which may be used in a higher controller layer to control acceleration or to determine the step size if discrete sampling is being used.

Figure 14. Visualization of the intersecting Vectors Technique

## 4.5 Non-linear Techniques

The previous 4 techniques use averaging of all sensor readings.  In contrast to averaging, the readings from the individual roots can be weighted differently based on some criteria. For example, it could weigh the robot readings such that only the minimum reading, the maximum reading, or the median reading is used. This selective weighting is the central premise to developing a primitive to traverse the separatrix line.

To traverse the separatrix line, the vector sum of the robot vector readings is calculated, and the resultant vector is compared to each robot via a cosine similarity score. The vector reading with the lowest cosine similarity score is then chosen as the direction to follow.

# # 5. Simulation Results and Discussion

Previous investigation into vector field navigation demonstrated success in navigating towards and around local extrema. These results are summarized in table 2.

Table 2: Regions of interest that are demonstrated in the Initial Exploration Paper

| ROI | Vector Field |
| --- | --- |
| Extrema Points | Sinks Source Fixed Vortex Sink + Vortex Superimposed Streams |
| Fastest Flow Stream | Source in Uniform Flow |

By using the direction follower technique, the results of [14] were also able to be replicated (see figure 15).

Figure 15: Seeking Extrema in sinking Vortex using direction follow technique

By using the vector to scalar technique, the local extreme could be located, particularly in sinks and sources or flows that super impose them, such as a sinking vortex. This can be seen in figure 16.

Figure 16: Seeking Extrema in sinking Vortex using mag technique

The ‘Vector to Scalar’ technique could also be used to orbit the extrema point by moving perpendicular to the direction of steepest ascent (see figure 17).

Figure 17: Orbiting Sinking Vortex using Vector to Scalar

Previous work has also demonstrated that these primitives can then be used sequentially to perform missions, such as the Fastest Flow Finder Technique. In that mission, they used only direction readings to reconfigure the cluster shape into a straight line, then use magnitude information only to navigate to the fastest flow in a stream moving along 1 axis only.

In contrast to this approach, this work demonstrates the ability to travel to the area of fastest flow without needing to change the cluster shape, nor without the need to use multiple primitives together in a mission. Figure 20 demonstrates the ability to reach the fastest flow of a Rankine Vortex using the vector sum technique.

Figure 20: Fastest Flow of Rankine Vortex

The vector to scalar technique was also used to find the stagnation point. By travelling perpendicular to the angle of steepest ascent, the robot cluster follows a curve that intersects with stagnation point.

Figure 18: Find Stagnation Point in spring in stream.

The nonlinear approach following the vector with the smallest cosine similarity score allowed the robot to traverse the separatrix line. This was demonstrated to work on separatrices associated with dipoles, springs in uniform flows, vortices in streams, and co-rotating vertices.

Figure 19: Maneuver Separatrix Line of a dipole

The intersecting vector technique was only able to locate extrema in simple fields. Often, there were cases where the vectors were parallel and didn’t intersect, or the intersecting points might not all be in the forward direction. Perhaps this technique could have some threshold for closeness of intersecting points when deciding whether it should be used.

# #6. Conclusions and Future Work

This work successfully models complex vector fields using simpler ones and demonstrates the ability of newly derived primitives to seek newly identified regions of interest using 3 robot clusters. The list of regions of interest in table 1 is not exhaustive, and new features may be identified in the future. This work may be extended into 3-dimensional vector fields, which may also have additional features of interest.

These techniques all involve using a centralized architecture. It would be worth extending these capabilities to non-centralized control techniques such as swarms.

The robustness of these algorithms to noise have not been analyzed. It would be interesting to test them not only with noisy environmental readings, being from the environment or from the sensor readings, but also with non-static environments. The controllers being used also maintain perfect formation. It would be interesting to check if the formation isn’t perfect. It could be worth analyzing the performance changes with different formations and different numbers of robots. This includes redundancy to noise rejection.

Work has been done to create a physical test bed to test robot primitives using RGB cameras and a multicolored floor map. The study concluded that there was not enough resolution between the color channels to be an effective test bed. Testing these primitives on a physical test bed is crucial for evaluating their real-world performance, and work is being done to modify the test bed to enable physical experiments.

# #7. ACKNOWLEDGEMENTS

This research would not be possible without the funding from Santa Clara University’s Robotic Systems Laboratory. Special thanks to Dr. Kitts for his guidance, encouragement, and technical assistance.

# #8. References

[1] Gradient-Based Cluster Space Navigation for Autonomous Surface Vessels Thomas Adamek, Member, IEEE, Christopher A. Kitts, Senior Member, IEEE, and Ignacio Mas, Member, IEEE

[2] C. A. Kitts and I. Mas, "Cluster Space Specification and Control of Mobile Multirobot Systems," in IEEE/ASME Transactions on Mechatronics, vol. 14, no. 2, pp. 207-218, April 2009, doi: 10.1109/TMECH.2009.2013943.

[3] I. Mas, S. Li, J. Acain, and C. A. Kitts, “Entrapment/escorting and patrolling missions in multi-robot cluster space control,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst., St. Louis, MO, USA, Oct. 2009, pp. 5855– 5861.

[4] M. Woolsey, C. A. Kitts and M. A. Neumann, "Gradient-based Adaptive Navigation of Multiple Diving Autonomous Surface Vehicles," OCEANS 2022, Hampton Roads, Hampton Roads, VA, USA, 2022, pp. 1-8, doi: 10.1109/OCEANS47191.2022.9977006.

[5] C. A. Kitts and M. Egerstedt, “Design, control, and applications of real world multirobot systems [from the Guest Editors],” IEEE Robot. Autom. Mag., vol. 15, no. 1, p. 8, Mar. 2008.

[6] Lee, Robert K., et al. "3-d adaptive navigation: Multirobot formation control for seeking and tracking of a moving source." 2020 IEEE Aerospace Conference. IEEE, 2020.

[7] S. T. Hart and C. A. Kitts, "Unifying Control Architecture for Reactive Particle Swarms," in IEEE/ASME Transactions on Mechatronics, vol. 28, no. 2, pp. 873-883, April 2023, doi: 10.1109/TMECH.2022.3211155.

[8] Hart, Shae T., Jake Kamenetsky, and Christopher A. Kitts. "Dynamic Elliptical Shaping Control for Swarm Robots." IEEE Access 11 (2023): 17454-17470.

[9] McDonald, Robert T., et al. "Navigation of scalar fronts with multirobot clusters in simulation and experiment." IEEE Systems Journal 14.3 (2020): 3755-3766.

[10] C. A. Kitts, R. T. McDonald, and M. A. Neumann, “Adaptive navigation control primitives for multirobot clusters: Extrema finding, contour following, ridge/trench following, and saddle point station keeping,” IEEE Access, vol. 6, pp. 17 625–17 642, 2018.

[11] Trakas, Panagiotis S., Andreas Tantoulas, and Charalampos P. Bechlioulis. "Formation Control of Nonlinear Multi-Agent Systems with Nested Input Saturation." Applied Sciences 14.1 (2023): 213.

[12] Matveev, Alexey, et al. "Non-Gradient Based Tracking of Environmental Field Isolines in a Changing Medium by Dubins-Vehicle Type Robots." IEEE Transactions on Automatic Control (2023).

[13] Cooper, Ryan, et al. "Initial Study of Multirobot Adaptive Navigation for Exploring Environmental Vector Fields." 2019 IEEE Aerospace Conference. IEEE, 2019.

[14] C. A. Kitts, R. T. McDonald, and M. A. Neumann, “Adaptive navigation control primitives for multirobot clusters: Extrema finding, contour following, ridge/trench following, and saddle point station keeping,” IEEE Access, vol. 6, pp. 17 625–17 642, 2018.

[15] I. Mas, S. Li, J. Acain, and C. A. Kitts, “Entrapment/escorting and patrolling missions in multi-robot cluster space control,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst., St. Louis, MO, USA, Oct. 2009, pp. 5855– 5861.

[16] Wang, J., Guo, Y., Fahad, M., and Bingham, B., 2019, “Dynamic Plume Tracking by Cooperative Robots,” IEEE/ASME Transactions on Mechatronics, early access.

[17] Liddy, Tommie, et al. "Obstacle avoidance using complex vector fields." Proc. of the 2008 Australasian Conference on Robotics and Automation, Canberra, Australia. 2008. – Single Robot with artificial vector field.

[18] Nelson, Derek R., et al. "Vector field path following for miniature air vehicles." IEEE Transactions on Robotics 23.3 (2007): 519-529.

[19] Hu, Bin-Bin, et al. "Spontaneous-Ordering Platoon Control for Multirobot Path Navigation Using Guiding Vector Fields." IEEE Transactions on Robotics (2023).

[20] Braquet, Martin, and Efstathios Bakolas. "Vector field-based collision avoidance for moving obstacles with time-varying elliptical shape." IFAC-PapersOnLine 55.37 (2022): 587-592.

[21] Yao, Weijia, et al. "Singularity-free guiding vector field for robot navigation." IEEE Transactions on Robotics 37.4 (2021): 1206-1221.

[22] Rajabhandharaks, Danop, et al. "Indoor Testbed for Vector Field Multirobot Adaptive Navigation: Feasibility Study." International Design Engineering Technical Conferences and Computers and Information in Engineering Conference. Vol. 59292. American Society of Mechanical Engineers, 2019.

[23] Matveev, Alexey, et al. "Gradient-free tracking of unsteady environmental level sets in dynamic environments by a nonholonomic robot." IFAC-PapersOnLine 53.2 (2020): 9256-9261.

[24] Garg, Kunal, and Dimitra Panagou. "Finite-time estimation and control for multi-aircraft systems under wind and dynamic obstacles." Journal of Guidance, Control, and Dynamics 42.7 (2019): 1489-1505..

[25] Thangavelautham, Jekan, and Yinan Xu. "Co-Evolution of Multi-Robot Controllers and Task Cues for Off-World Open Pit Mining." arXiv preprint arXiv:2009.09149 (2020).

[26] Kalita, Himangshu, et al. "Guidance, navigation and control of multirobot systems in cooperative cliff climbing." arXiv preprint arXiv:1701.07553 (2017)

[27] R. K. Lee, C. A. Kitts, M. A. Neumann and R. T. Mcdonald, "Multiple UAV Adaptive Navigation for Three-Dimensional Scalar Fields," in IEEE Access, vol. 9, pp. 122626-122654, 2021, doi: 10.1109/ACCESS.2021.3107854.